# EEL 6878 Homework 4

Dylan D’Silva

# Question 1.

The plot below shows the Graph Fourier Transform of the Heat filtered random signals.

Figure 1: GFT of Heat Filtered Random Graph Signals

The graph signal generated by applying a Heat filter bank with scale=10 is a low-pass signal. Low frequencies see little to no attenuation, while high frequencies see reduction in power by orders of magnitude.

When applying the Heat kernel with scale=0 to the random signal, the resulting signal had a smoothness of 580.9577. For the case when scale=10, the resulting signal had a smoothness of 0.7838.

The higher scale filter produced a significantly smoother signal, which can be observed in the top row of Figure 1. Transitions between node values at scale=10 appear much more gradual than for scale=0. Higher frequencies present in the signal are responsible for more drastic changes in the values of connected nodes. This allows for a rougher signal with more variation. Attenuating the signal at these frequencies removes most of the potential for large changes between nodes. A Heat filter with scale=10 does more to eliminate higher frequencies in the signal, as seen in the second row of Figure 1. Higher frequency content remains present in the filtered signal for scale=0 while being attenuated by orders of magnitude when scale=10.

# Question 2.

## Part 1) Loaded Graphs and Their Eigenvalues

The Erdős-Rényi graph has frequencies that increase steadily. The Ring graph has frequencies which follow that of a circulant graph. The graph created by the Cora dataset has many very low frequencies. From the plots, the Cora graph has several connected components, meaning several of the Laplacian eigenvalues will be 0. The several other low frequencies indicate the graph or its components have low connectivity. This makes sense from how many nodes and components the graph and dataset contain.

Figure 2: Generated or Loaded Graphs and Spectral Content

## Part 2) Graph Eigenvectors

The Cora graph contains several connected components, so λ2=λ3=λ6=0. That being said, each of the eigenvectors correspond to a different connected component set of components.

## Embedding and Labelling

After performing the t-SNE and applying K-means clustering for labeling, the resulting labels achieved an ARI of 0.109 and a Silhouette score of 0.378.

## Ring Denoising

The noisy signal in the Ring graph and its spectral content are shown below. Low frequency content is prominent due to the construction of the original signal. Higher frequency content is present due to the introduction of random noise.

## Denoised Signal

# Question 3.

## Training Loss

The plot below shows the training loss across 200 training epochs. The training loss shows a steep decline towards zero, followed by a plateau at lower magnitudes. It seems that most improvements are made in early epochs. At later epochs, the model is making incremental improvements.

## Model Evaluations

The plot below shows the node labels according to the Cora dataset, the labels as predicted by the GCN, and the disagreement between the two values. The smaller blue nodes are nodes for which labels that disagree. Larger red nodes are nodes for which the labels disagree.  The GCN is able to correctly predict the labels for a vast majority of the nodes.